Find out all the Geometry Classes to Play with Points, Lines, Curves, Surfaces, Solids, Geometries etc

To access all these classes in revit tool we need to import all the classes using:

from Autodesk.DesignScript.Geometry import *

This will give the access to different geometry classes :

Point, Vector, Line, Arc, Circle, NurbsCurve, PolyCurve , Surface ,Solid, Plane CoordinateSystem , BoundingBox , Mesh, Polygon, Cuboid, Sphere, Cone, Cylinder

Remember:  all these geometry classes are in designScript library

POINT: A Point represents a location in 3D space.

Root

└── Geometry

└── Point

- ByCartesianCoordinates

- Normally points are created in the global coordinate system.

- This method creates a point relative to another coordinate system.

| Parameter | Meaning |
| --- | --- |
| cs | Coordinate System |
| x | X distance |
| y | Y distance |
| z | Z distance |

cs = CoordinateSystem.Identity()

p = Point.ByCartesianCoordinates(cs, 10, 5, 0)

- ByCoordinates(x, y): Creates a point in the XY plane.

- Automatically assumes z=0

p = Point.ByCoordinates(5, 10) #(5,10,0)

- ByCoordinates(x, y, z): Creates a full 3D point.(mostly used)

p = Point.ByCoordinates(5, 10, 20)

- ByCylindricalCoordinates: Creates a point using cylindrical coordinates.

- Instead of (x,y,z) we use (radius, angle, height)

| radius | Distance from center |
| --- | --- |

| angle | Rotation angle |
| --- | --- |

| height | Z elevation |
| --- | --- |

Point.ByCylindricalCoordinates(cs, radius, angle, height)

- BySphericalCoordinates: Creates a point using spherical coordinates.

- Instead of XYZ we use (radius, horizontal angle, vertical angle)

p = Point.BySphericalCoordinates(cs, 10, 45, 60)

- Origin: prints origin

- Origin is the reference point of the entire model space.

origin = Point.Origin()

- Add: Adds a vector to a point.

- Equivalent to moving/translating the point.

- Point + Vector = New Point

point.Add(vector)

Example:

p = Point.ByCoordinates(1,2,3)

v = Vector.ByCoordinates(5,0,0)

newPoint = p.Add(v)

- AsVector: Converts a point into a vector.

| Point | Vector |
| --- | --- |
| Location | Direction |
| Has position | Has magnitude/direction |

point.AsVector()

Example:

p = Point.ByCoordinates(2,3,4)

v = p.AsVector()

- Project: projects geometry onto something along a direction vector.

projected = point.Project(surface, vector)

- PruneDuplicates: Removes duplicate points.

cleanPoints = Point.PruneDuplicates(points, 0.01)

- Subtract: Moves a point backward using a vector.

point.Subtract(vector)

- X: Returns X coordinate.(similarly for y and z)

point.X

point.Y

point.Z

CURVE: A curve is ANY path in geometry.

Root

└── Geometry

└── Curve

├── Line

├── Arc

├── Circle

├── Ellipse

├── NurbsCurve

└── PolyCurve

- ByBlendBetweenCurves: Creates a smooth transition curve between two curves.

Curve.ByBlendBetweenCurves(curve1, curve2)

(point+point=line

Line+line=curve)

- ByIsoCurveOnSurface: Creates a curve on a surface using UV directions.

- Surfaces have 2 directions: U direction ,V direction Like latitude/longitude.

Curve.ByIsoCurveOnSurface(surface, direction, parameter)

| Parameter | Meaning |
| --- | --- |

| surface | Input surface |
| --- | --- |

| direction | "U" or "V" |
| --- | --- |

| parameter | Position on surface |
| --- | --- |

- ByParameterLineOnSurface: Creates a line directly on a surface using UV coordinates.

Curve.ByParameterLineOnSurface()

- ApproximateWithArcAndLineSegments: Converts complex curves into arcs, straight lines

approx = curve.ApproximateWithArcAndLineSegments()

- CoordinateSystemAtParameter: Creates a local coordinate system on the curve.

curve.CoordinateSystemAtParameter(t)

- CoordinateSystemAtSegmentLength: same as above but here instead of parameter we use “distance along curve ” is used

curve.CoordinateSystemAtSegmentLength( )

- StartParameter: returns 0 (usually)

- EndParameter: returns 1 (may vary for different curve types).

- Extend: lengthens a curve.(works only on open curves)

- ExtendStart: Extends only start side.

- ExtendEnd: extends only end side.

- Extrude: (turns 2D curve into 3D geometry)

- Extrude(curve, distance): Uses normal direction automatically.

Curve.Extrude(curve, distance)

- Extrude(curve, direction): Uses vector direction.

Curve.Extrude(curve, direction)

- Extrude(curve, direction, distance): Most controllable version.

- ExtrudeAsSolid:  Same as extrusion, but creates CLOSED solid.

- ExtrudeAsSolid (curve, distance): Extrudes a Curve in the Normal direction by the specified distance. Curve must be closed.

- ExtrudeAsSolid (curve, direction): Extrudes a Curve in the specified direction, by the length of the input Vector. Curve must be closed.

- ExtrudeAsSolid (curve, direction, distance): Extrudes a Curve in the specified direction, by the specified distance. Curve must be closed.

- HorizontalFrameAtParameter: Creates a horizontal coordinate system on curve.

- Join: Joins curves into PolyCurve.( Line + Arc + Line)

- NormalAtParameter: Returns perpendicular vector.

- TangentAtParameter: Returns direction vector along curve.

- Offset: Creates parallel curve.

- ParameterAtPoint: Gets parameter value from point.

- PointAtParameter: It gives a point ON the curve at a specific parameter value.

curve.PointAtParameter(t)

- PointAtSegmentLength: Gets point at specific distance along curve.

- PointsAtEqualChordLength: Divides curve equally. Get the parameter at a particular chord length along the curve from given location.

- PointsAtEqualSegmentLength: Uses true curve length. Get the parameter at a particular arc length along the curve.

- Patch:Patch a closed Curve

- PLANES:

- PlaneAtParameter: Returns a Plane whose normal aligns with the tangent of the Curve. Parameters are adjusted such that 0 is always the start Point and 1 is always the end Point.

- PlaneAtSegmentLength: Returns a Plane at the specified distance along the Curve from the start Point. The normal of the Plane aligns with the tangent of the Curve.

- Project: Project another piece of Geometry onto this along a given direction Vector.

- PullOntoPlane: Create a curve by pulling onto plane

- PullOntoSurface: Pull this Curve onto the input Surface, in the direction of the Surface normals.

- Reverse: Reverse the direction of the curve.

- SegmentLengthAtParameter: Get the arc length along the Curve at the particular parameter.

- SegmentLengthBetweenParameters: Get the arc length between two parameter points on the curve.

- Simplify: Returns a new Curve approximated with the supplied tolerance.

- SplitByParameter (curve, parameter): Split a Curve into two pieces at the given parameter.

- SplitByPoints: Split a Curve into multiple pieces at the given points

- StartParameter: Get the start of the domain in which the curve can be evaluated

- SweepAsSolid: Sweeps this closed Curve along the path Curve, creating a Solid

- SweepAsSurface: Sweeps this Curve along the path Curve, creating a Surface

- TangentAtParameter: Get a Vector tangent to the curve at a specified parameter between StartParameter() and EndParameter().

- ToNurbsCurve: Converts the Curve to a NurbsCurve approximation

- TrimByEndParameter: Removes the end of the Curve at the specified parameter

- TrimByParameter: Removes the beginning and end of the Curve at the specified parameters.

- TrimByStartParameter: Removes the start of the Curve at the specified parameter

- TrimInteriorByParameter: Removes the interior portion of a Curve at the specified parameters

- TrimSegmentsByParameter (curve, parameters): Removes several segments of the curve, discarding the 1st, 3rd, 5th ... segments

- TrimSegmentsByParameter (curve, parameters, discardEvenSegments): Removes several segments of the curve, discarding the 1st, 3rd, 5th ... segments

LINES: In Dynamo, a Line is a type of Curve.

Root

└── Geometry

└── Curve

└── Line

- ByBestFitThroughPoints: Creates a line that best fits a group of scattered points

- This method creates a line that passes as close as possible to all points. This line is called best fit line or regression line.

- The line may NOT pass exactly through all points.

- It tries to minimize overall error.

from Autodesk.DesignScript.Geometry import *

pts = [

Point.ByCoordinates(0,0,0),

Point.ByCoordinates(1,1,0),

Point.ByCoordinates(2,2.2,0),

Point.ByCoordinates(3,3.1,0)

]

line = Line.ByBestFitThroughPoints(pts)

- ByStartPointDirectionLength:

Creates a line:

- starting from a point

- moving in a vector direction

- for a specific length

Line.ByStartPointDirectionLength(startPoint, vector, length)

| Parameter | Meaning |
| --- | --- |

| startPoint | Starting location |
| --- | --- |

| vector | Direction |
| --- | --- |

| length | Distance |
| --- | --- |

- ByStartPointEndPoint: Creates a line between two points.

Line.ByStartPointEndPoint(start, end)

- ByTangency: Creates a tangent line on a curve.

Line.ByTangency(curve, parameter)

- Direction: Returns the direction vector of the line.

Di=line.Direction

SURFACE: a thin skin or sheet existing in 3D space

- ByLoft (crossSections): Creates a surface between curves.

Example:

from Autodesk.DesignScript.Geometry import *

c1 = Circle.ByCenterPointRadius(

Point.ByCoordinates(0,0,0),  5

)

c2 = Circle.ByCenterPointRadius(

Point.ByCoordinates(0,0,10),

8

)

surface = Surface.ByLoft([c1,c2])

OUT = surface

- Surface.ByLoft(crossSections, guideCurves): Same loft, but guide curves control shape.

- Surface.ByPatch: Create a Surface by filling in the interior of a closed boundary defined by input Curves.

- Surface.ByPerimeterPoints: Creates surface directly from perimeter points.

Example:

pts = [

Point.ByCoordinates(0,0,0),

Point.ByCoordinates(10,0,0),

Point.ByCoordinates(10,10,0),

Point.ByCoordinates(0,10,0)

]

surface = Surface.ByPerimeterPoints(pts)

- Surface.ByRevolve: Rotates curve around axis.

Example:

line = Line.ByStartPointEndPoint(

Point.ByCoordinates(5,0,0),

Point.ByCoordinates(5,0,10)

)

surface = Surface.ByRevolve(

line,

Point.ByCoordinates(0,0,0),

Vector.ZAxis(),

0,

360

)

- Surface.ByRuledLoft: Create a Surface by lofting between input cross section Lines. This is slightly faster and produces a less smooth result than Surface.ByLoft.

- Surface.BySweep: Create a Surface by sweeping a cross section Curve along a path.

Example:

profile = Circle.ByCenterPointRadius(

Point.ByCoordinates(0,0,0),

2

)

path = NurbsCurve.ByPoints([

Point.ByCoordinates(0,0,0),

Point.ByCoordinates(10,5,5),

Point.ByCoordinates(20,0,10)

])

surface = Surface.BySweep(profile,path)

- Surface.BySweep2Rails: Sweep the cross section curve along a path guided by a two rails

- ApproximateWithTolerance: Converts surface to approximated NURBS.

- CoordinateSystemAtParameter: Return a CoordinateSystem aligned with principal curvature directions.

- CurvatureAtParameter: Measures surface bending.

- DerivativesAtParameter: Returns mathematical derivatives and Advanced geometry analysis.

- FlipNormalDirection:  Returns a new Surface with the Normal flipped. Leaves this surface unchanged.

- GaussianCurvatureAtParameter: Returns the Gaussian curvature at U and V parameters.

- GetIsoline: Creates U or V isolines. Equivalent to:surface grid lines

- Join: Combines this Surface and input Surface into a PolySurface

- NormalAtParameter: Return the normal Vector at specified U and V parameters.

- NormalAtPoint: Return the surface normal at the input Point on the Surface.

- Offset: Offset Surface in direction of Surface normal by specified distance.

- PerimeterCurves: Return all the boundary Curves of the Surface.

- PointAtParameter: Return the Point at specified U and V parameters.

- PrincipalCurvaturesAtParameter: Returns the principal curvatures at the U and V parameters.

- PrincipalDirectionsAtParameter: Returns principal direction vectors at U and V parameters.

- ProjectInputOnto: Projects the input Geometry onto this Surface in the input Vector direction

- SubtractFrom: Subtract the input tools from this Surface.

- TangentAtUParameter: Return the U tangent Vector at specified U and V parameters.

- TangentAtVParameter: Return the V tangent Vector at specified U and V parameters.

- Thicken: Converts surface into solid.

- Thicken (surface, thickness): Thicken Surface into a Solid, extruding in the direction of Surface normals on both sides of the Surface.

- Thicken (surface, thickness, both_sides): Thicken Surface into a Solid, extruding in the direction of Surface normals. If both_sides parameter is true, surface is thickened on both sides.

- ToNurbsSurface: Gets a Nurbs representation of the Surface. This method may approximate Surface in certain circumstances.

- TrimWithEdgeLoops: Trim the surface with a collection of loops within the surface

- UVParameterAtPoint: Return the UV parameter pair at the input Point. This is the inverse of Point at parameter.

- Area: Returns the total surface area.

- Closed: Returns true if the Surface is closed in U or V directions

- ClosedInU: Returns true if the Surface is closed in U direction.

- ClosedInV: Returns true if the Surface is closed in V direction

- Perimeter: Returns the sum of all edges of the Surface.

SOLID: a closed 3D object having volume

- ByJoinedSurfaces: Create a solid by specifying it's component faces as Surfaces.

- ByLoft (crossSections): Create a Solid by lofting between input cross section closed Curves.

- ByLoft (crossSections, guideCurves): Create a Solid by lofting between input cross section closed Curves.

- ByRevolve: Rotates profile around axis to create solid.

- BySweep: Sweep a closed Curve along a path.

- BySweep2Rails: Sweep a closed profile Curve along two rail Curves.

- ByUnion: Union a collection of solids into one solid

- Centroid: The centroid of the Solid

- Chamfer: Cuts sharp edges diagonally. Chamfers a Solid along input Edges with a given offset from the edge corner.

- Difference: The boolean difference of this Solid with another

- DifferenceAll: Subtracts union of many solids.

- Fillet: Rounds sharp edges. Fillets a Solid along input Edges with a given radius.

- ProjectInputOnto: Projects geometry onto solid. Projects the input Geometry onto this Solid, in the direction of the input Vector

- ThinShell: Obtain a solid Shell from the Faces of this Solid

- Union: The boolean union of this Solid and another.

- Area: Returns the surface area -- sum of all the areas of all faces

- Volume: The total volume of the Solid

GEOMETRY: In Dynamo, Geometry is the base parent class for almost all geometry types.

Root

└── Geometry

├── Point

├── Curve

├── Surface

├── Solid

└── ...